Sample Wiring

- Is a smaller sample or a larger sample better for resistance measurements (suppose you lock the aspect ratio)?

- When you apply magnetic field (out-of-plane direction), is the change in resistance magnetoresistance or 1/magnetoconductance?

Classically, according to Drude transport (see Ashcroft and Mermin Ch. 1)

ρxx=1neμ=constant and 1/σxx=(1+μ2B2)1neμ

Is the measured R proportional to ρxx or R proportional to 1/σxx?

(a)

(b)

Draw how the current flows in the following geometry when the magnetic field is applied out-of-plane.

- In (a), what happens if you put the V+ and V- closer?

- In (b), what happens if the size of the voltage contacts is bigger?

Lock-in Measurements

- Setup a two-terminal resistance measurement configuration.

- Complete the two-terminal measurements.

Check if I and V scale linearly, frequency dependence.

| Ω | 4 | 3 | 2 |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |

- What is the origin of the two terminal resistances? Would you continue the experiment if any of the resistance values are significantly larger or non-Ohmic?

- Setup the lock-in for a four-terminal resistance measurement.

- What is R3, 4; 5, 6 = V5,6/I3,4 ? Check if I and V scale linearly, check frequency dependence.

- If you are going to measure R vs T or R vs H, what current, frequency, time constant, sensitivity, etc settings will you choose?

- If you choose a time constant that is too short, what happens? Are the fluctuations noise? Try changing the excitation frequency and see what happens.

- Measure R3,4, 6,5 ? Is it the same as -R3,4;5,6? If not, what can be the possible reason?